# Pico_lib Documentation

# Contents

device = pico_setup(Serial_No)

pico_enable_channel(device, channel, y_range)

pico_set_trigger(device, channel)

[interval, maxSamples] = pico_timebase(device)

[timebase, chA, chB] = pico_single_shot(device, acquisitionLength, interval,  maxSamples)

[timebase, chA, chB] = pico_averaged_waveform(device, acquisitionLength, interval,  maxSamples)

# Function Definitions

## pico_setup

Syntax:

device = pico_setup('IU908/0031');

Description:

Argument is the serial number of the device you want to connect to, found underneath the barcode on the underside of the picoscope between two asterisks.

device is an icdevice object which is passed to all other functions when controlling the scope. This setup allows the control of multiple scopes.

## pico_enable_channel

Syntax

pico_enable_channel(device, 0, 2);

Description

This function turns on channels ready for acquisition and configures the settings of that channel. The first argument is the device outputted from pico_setup, the second is the channel number to be switched on (starting from 0), and the final argument is the y range of the channel. The function will automatically find the closest available y range to the one requested and set it to that. Allowed y ranges are [10E-3, 20E-3, 50E-3, 100E-3, 200E-3, 500E-3, 1, 2, 5, 10] in Volts.

The function defaults to AC coupling and no analogue offset, but these can be configured by editing the function if desired.

## pico_set_trigger

Syntax:

pico_set_trigger(device, channel);

Description

Configures triggering on the rising edge of channel channel.

## pico_timebase

Syntax:

[interval, maxSamples] = pico_timebase(device);

Description

Sets the timebase to the max time resolution and returns the interval between samples in nanoseconds and the maximum number of samples in a single acquisition, maxSamples. These two output parameters are then passed to whichever function you use to read the data.

## pico_single_shot

Syntax:

[timebase, chA, chB] = pico_single_shot(device, acquisitionLength, interval,  maxSamples);

Description:

Used for getting a single pair of waveforms from the picoscope with no averaging. Takes the device object, acquisitionLength (time, in seconds, from trigger to the last point you want to capture), and interval and maxSamples from pico_timebase. Returns the timebase in seconds, and the two channels’ voltages in Volts.

## pico_averaged_waveform

Syntax:

[timebase, chA, chB] = pico_averaged_waveform(device, numAvg, acquisitionLength, interval, maxSamples);

Description:

Works exactly the same as pico_single_shot, only takes and extra argument, numAvg (the number of averages to be taken). It will then run an optimized data collection routine to collect that many averages (or possibly slightly more if it will be faster), and return the results.